ANNEX 7

POLREP POLLUTION REPORTING SYSTEM

November 2017

POLREP POLLUTION REPORTING SYSTEM

1.	The pollution reporting system is for use between the Contracting Parties to the Prevention and Emergency Protocol to the Barcelona Convention themselves and between the Contracting Parties and the Regional Centre, for exchanging information when pollution of the sea has occurred or when a threat of such is present.

2.	The POLREP is divided into three parts:

.1	Part I or POLWARN (POLlution WARNing), comprising figures 1-5, gives first information or warning of the pollution or the threat

.2	Part II or POLINF (POLlution INFormation), comprising figures 40-60, gives detailed supplementary report, as well as situation report.

.3	Part III or POLFAC (POLlution FACilities), comprising figures 80-99, is used for requesting assistance from other Contracting Parties and for defining operational matters related to the assistance

3.	The division into three parts is only for identification purposes.  For this reason consecutive figures are not used.  This enables the recipient to recognize merely by looking at the figures whether he is dealing with part I (1-5), part II (40-60) or part III (80-99).  This method of division shall in no way exclude the use of all figures in a full report or the separate use of single figures from each part or the use of single figures from different parts mixed in one report.

4.	Part II is the logical consequence of part I.  Having transmitted part I, the Party concerned can inform the other Parties of its assessment of the nature and extent of the incident by using the appropriate figures from part II.

5.	Part III is for the request for assistance and related matters exclusively.  Detailed description of the format for requesting assistance within the framework of the present Plan is given in Annex 8.

6.	A summarized list of POLREP is given below.

Address               from ....               to ....

INTRODUCTORY		Date Time Group

PART				Identification

Serial number

__________________________________________________________________________________________

1	Date and time

2	Position

PART I				3	Incident

(POLWARN)			4	Outflow

5	Acknowledge

__________________________________________________________________________________________

40	Date and time

41	Position

42	Characteristics of pollution

43	Source and cause of pollution

44	Wind direction and speed

45	Current or tide

46	Sea state and visibility

PART II				47	Drift of pollution

(POLINF)			48	Forecast

49	Identity of observer and ships on scene

50	Action taken

51	Photographs or samples

52	Names of other States informed

53-59	Spare

60	Acknowledge

__________________________________________________________________________________________

80	Date and time

81	Request for assistance

PART III			82	Cost

(POLFAC)			83	Pre-arrangements for the delivery

84	Assistance to where and how

85	Other States requested

86	Change of command

87	Exchange of information

88-98	Spare

99	Acknowledge

EXPLANATION OF A POLREP MESSAGE

INTRODUCTORY PART

| CONTENTS | REMARKS |
| --- | --- |
| ADDRESS | Each report should start with an indication of the country whose competent national authority is sending it and of addressee e.g.: FROM: ISR (indicates the country which sends the report) TO GRE (indicates the country to which it is sent) or REMPEC (indicates that the message is sent to the Regional Centre. |
| DTG (Day Time Group) | The day of the month followed by the time (hour and minute) of drafting the message. Always a 6-figure group which may be followed by month indication. Time should be stated either as GMT, e.g. 092015Z june (i.e. the 9th of the relevant month at 20.15 GMT) or as local time e.g. 092115LT june. |
| IDENTIFICATION | - "POL..." indicates that the report might deal with all aspects of pollution (such as oil as well as other harmful substances). - ".....REP" indicates that this is a report on a pollution incident. - It can contain up to 3 main parts: - Part I (POLWARN) - is an initial notice (a first information or a warning) of a casualty or the presence of oil slicks or harmful substances. This part of the report is numbered from 1 to 5. - Part II (POLINF) - is a detailed supplementary report to Part I. This part of the report is numbered from 40 to 60. - Part III (POLFAC) - is for requests for assistance from other Contracting Parties, as well as for defining operational matters related to the assistance. This part of the report is numbered from 80 to 99 (see Annex 6). - BARCELONA CONV indicates that the message is sent within the framework of the Prevention and Emergency Protocol to the Barcelona Convention. - CGI SCP indicates that the message is sent within the framework of the Sub-regional Marine Pollution Contingency Plan for Cyprus, Greece and Israel. - Parts I, II and III can be transmitted all together in one report or separately. Furthermore, single figures from each part can be transmitted separately or combined with figures from the two other parts. - Figures without additional text shall not appear in the POLREP. - When Part I is used as warning of a serious threat, the message should be headed with the traffic priority word "URGENT". - All POLREPs containing ACKNOWLEDGE figures (5, 60 or 99) should be acknowledged as soon as possible by the competent national authority of the country receiving the message. - POLREPs should always be terminated by a message from the reporting State, which indicates that no more operational communication on that incident can be expected. |

| SERIAL NUMBER | It must be possible to identify each POLREP and the person who receives it must be able to check whether all reports of that particular incident have been received. This is done by using standard ISO 3166-1 alpha-3 country codesa: Cyprus CYP Greece GRC Israel ISR REMPEC REMPEC - The nation-identifier should be followed by a stroke and the name of the ship or other installation involved in the accident and another stroke followed by the number of the actual report concerning this particular incident. - ISR/POLLUX/1 indicates that this is the first report from Israel concerning the incident of MT "Pollux". - ISR/POLLUX/2, in accordance with the described system, indicates the second report on the same incident. - The last and final POLREP will show as follows: ISR/POLLUX/5 FINAL, which means that this is the fifth and final report from Israel concerning the incident of MT "Pollux". - Three letter country codes (ISO alpha-3) for other Contracting Parties of the Prevention and Emergency Protocol to the Barcelona Convention are as follows: Albania ALB Algeria DZA Bosnia & Herzegovina BIH Croatia HRV Egypt EGY The EU EU France FRA Italy ITA Lebanon LBN Libya LBY Malta MLT Monaco MCO Montenegro MNE Morocco MAR Slovenia SVN Spain ESP Syria SYR Tunisia TUN Turkey TUR - When answering a POLREP the serial number used by the transmitting State is to be used as reference in the answer. However, it is not necessary for countries to adhere to the POLREP system in responding to POLREPs. |
| --- | --- |

Part I (POLWARN)

| CONTENTS | REMARKS |
| --- | --- |
| 1 DATE AND TIME | The day of the month as well as the time of the day when the incident took place or, if the cause of the pollution is not known, the time of the observation should be stated with 6 figures. Time should be stated as GMT for example, 091900z (i.e. the 9th of the relevant month at 1900 GMT) or as local time for example, 091900lt (i.e. 9th of the relevant month at 1900 local time) |
| 2 POSITION | Indicates the main position of the incident in latitude and longitude in degrees and minutes and may, in addition, give the bearing of and the distance from a location known by the receiver. |
| 3 INCIDENT | The nature of the incident should be stated here, such as BLOWOUT, TANKER GROUNDING, TANKER COLLISION, OIL SLICK, etc. |
| 4 OUTFLOW | The nature of the pollution, such as CRUDE OIL, FUEL OIL, etc. as well as the total quantity in tonnes of the outflow and/or the flow rate, as well as the risk of the further outflow. If there is no pollution but a pollution threat, the words NOT YET followed by the substance, for example, NOT YET FUEL OIL, should be stated. |
| 5 ACKNOWLEDGE | When this figure is used the message should be acknowledged as soon as possible by the competent national authority. |

Part II (POLINF)

| CONTENTS |  | REMARKS |
| --- | --- | --- |
| 40 | DATE AND TIME | No. 40 relates to the situation described in figures 41 to 60 if it varies from figure 1. |
| 41 | POSITION AND/OR EXTENT OF POLLUTION ON/ABOVE/IN THE SEA | Indicates the main position of the pollution in latitude and longitude in degrees and minutes and may in addition give the distance and bearing of some prominent landmark known to the receiver if other than indicated in figure 2. Estimate amount of pollution (e.g. size of polluted areas, number of tonnes of oil spilled if other than indicated in figure 4, or number of containers, drums etc. lost). Indicates length and width of slick given in nautical miles if not indicated in Fig. 2. |
| 42 | CHARACTERISTICS OF POLLUTION | Gives type of pollution, e.g. type of oil with viscosity and pour point. For all, give also appearance, e.g. liquid, liquid oil, semi-liquid sludge, tarry lumps, weathered oil, discolouration of sea, visible vapour. |
| 43 | SOURCES AND CAUSE OF POLLUTION | For example, from vessel or other undertaking. If from vessel, say whether as a result of a deliberate discharge or casualty. If the latter, give brief description. Where possible, give name, type, size, call sign, nationality and port of registration of polluting vessel. If vessel is proceeding on its way, give course, speed and destination. |
| 44 | WIND DIRECTION AND SPEED | Indicates wind direction and speed in degrees and m/s. The direction always indicates from where the wind is blowing. |
| 45 | CURRENT DIRECTION AND SPEED AND/OR TIDE | Indicates currents direction and speed in degrees and m/s. The direction always indicates the direction in which the current is flowing. |
| 46 | SEA STATE AND VISIBILITY | Sea state indicated as wave height in metres. Visibility in nautical miles. |
| 47 | DRIFT OF POLLUTION | Indicates drift course and speed of pollution in degrees and knots and tenths of knots. |
| 48 | FORECAST OF LIKELY EFFECT OF POLLUTION AND ZONES AFFECTED | For example, arrival on beach with estimated timing. Results of mathematical (computer) spill forecasting models should also be included under this figure. |
| 49 | IDENTITY OF OBSERVER/ REPORTER, IDENTITY OF SHIPS ON SCENE | Indicates who has reported the incident. If a ship, her name, home port, flag and call sign must be given. Ships on scene can also be indicated under this item by name, home port, flag and call sign, especially if the polluter cannot be identified and the spill is considered to be of recent origin. |
| 50 | ACTION TAKEN | Outlines any action taken in response to the pollution. |
| 51 | PHOTOGRAPHS OR SAMPLES | Indicates if photographs or samples from the pollution have been taken. Contact details (email address, telephone, fax numbers) of the sampling authority should be given. |
| 52 | NAMES OF OTHER STATES AND ORGANIZATIONS INFORMED |  |
| 53 - 59 |  | SPARE FOR ANY OTHER RELEVANT INFORMATION (e.g. results of sample or photographic analysis, results of inspection of surveyors, statements of ship's personnel, etc.) |
| 60 | ACKNOWLEDGE | When this figure is used the message should be acknowledged as soon as possible by the competent national authority. |

Part III (POLFAC)

| CONTENTS |  | REMARKS |
| --- | --- | --- |
| 80 | DATE AND TIME | No. 80 is related to the situation described below, if it varies figures 1 and/or 40. |
| 81 | REQUEST FOR ASSISTANCE | Type and amount of assistance required in form of: - specified equipment - specified equipment with trained personnel - complete strike teams - personnel with special expertise with indication of country from which it is requested. |
| 82 | COST | Requirements for cost information to requesting country of delivered assistance. |
| 83 | PRE-ARRANGEMENTS FOR DELIVERY OF ASSISTANCE | Information concerning customs clearance, access to the territorial sea, etc. in the requesting country. |
| 84 | TO WHERE ASSISTANCE SHOULD BE RENDERED AND HOW | Information concerning the delivery of the assistance, e.g. rendez-vous at sea with information on frequencies to be used, call sign and name of NOSC/SOSC of the requesting country, or land based authorities with contact details (email addresses, telephone and fax numbers and contact persons). |
| 85 | NAMES OF OTHER STATES AND ORGANIZATIONS | Only to be filled in if not covered by figure 81, e.g. if further assistance is later needed by other States. |
| 86 | CHANGE OF COMMAND | When a substantial part of an oil pollution or serious threat of oil pollution moves or has moved into the area of responsibility of another Party, the Party which has exercised the Operational Command may request the other Party to take over the Operational Command (cf. Article 3.1 of the Plan). |
| 87 | EXCHANGE OF INFORMATION | When a mutual agreement has been reached between two Parties on a change of Operational Command, the State transferring the Operational Command should give a report on all relevant information pertaining to the operation to the State taking over the command. |
| 88 - 98 |  | SPARE FOR ANY OTHER RELEVANT REQUIREMENTS OR INSTRUCTIONS |
| 99 | ACKNOWLEDGE | When this figure is used the message should be acknowledged as soon as possible by the competent national authority. |

EXAMPLE

(Full POLREP Report including Parts I, II and III)

Address						From:	CYP

To:	GRC, ISR and REMPEC

Date Time Group					181100z june

Identification						POLREP BARCELONA CONV/CGI SCP

Serial number						CYP/POLLUX/2

1	Date and time				1	181000z

2	Position					2	lat 3442'N; long 3353'E

3	Incident					3	Tanker collision

4	Outflow					4	Crude oil, estimated 3000 tonnes

__________________________________________________________________________________

41	Position and/or extent of		41	The oil is forming a slick 0.5 N miles long

pollution on/above/in sea			to the north-west.  Width up to 0.3 N miles.

42	Characteristics of pollution		42	Arabian Heavy crude.  Viscosity 18.9 cSt at 37.8C.

43	Source and cause of pollution		43	Liberian m tanker POLLUX of Monrovia, 21000 GRT, call sign xxx in collision with Honduran bulk carrier CASTOR of Puerto Cortes, 58000 GRT, call sign yyy.  Two tanks damaged in POLLUX.  No damage in CASTOR.

44	Wind direction and speed		44	110 - 10 m/s.

45	Current direction and speed		45	50 - 0.3 knots.

and/or tide

46	Sea state and visibility			46	Wave height up to 2 m.  10 Nautical miles.

47	Drift of pollution				47	320 - 0.95 knots.

48	Forecast of likely effects		48	Could reach Larnaca Bay, CYP, on 19 june

of pollution and zones affected

49	Identity of observer/reporter		49	CASTOR, figure 43 refers.

Identity of ships on scene

50	Action taken				50	NCP activated. Water intakes of a power plant and the refinery are being protected by booms.  A tug boat and a helicopter with dispersant spraying equipment on route to the area of the slick.

51	Photographs or samples		51	Photographs and video recording taken.

52	Names of other States and		52	REMPEC

organizations informed

53	Spare					53	Sub-regional Contingency Plan is activated.

Mr. C. Demetrides designated SOSC.

__________________________________________________________________________________

81	Request for assistance			81	GRC is requested for 1 surveillance aircraft with remote sensing equipment.  ISR is requested for 1 tug boat with 2000 m harbour boom and 2 disk type skimmers.

82	Cost					82	GRC and ISR are requested for an approximate cost rate per day of assistance rendered.

83	Pre-arrangements for the		83	GRC aircraft granted permission to enter CYP air space

delivery of assistance				for spill surveillance and to land at Larnaca for logistics, informing SOSC beforehand.  ISR equipment granted permission to enter CYP on temporary basis, without duties.

84	To where assistance should be		84	GRC aircraft at Larnaca airport. Contact SOSC by

rendered and how				radio, frequency xxx. ISR tug boat rendez-vous lat 3445'N; long 3350'E.  Prior to arrival report to SOSC on VHF channels 16 and 67.

99	Acknowledge				99	ACKNOWLEDGE

NOTE:  the text of POLREP message should not include parts shaded in grey.